We have prepared the CSV files:
i) Vehicle_units: https://drive.google.com/file/d/1r3PHO7v83zMnKadu2fEAhhsBjVHqfz3A/view

ii) queue_length:

https://drive.google.com/file/d/1_Vpytm3DKAV6Xo3IwOXcmY-LY1Lm-o3p/view

iii) OLD_Data_CSV_Vehicle_Count:  https://drive.google.com/file/d/1M4YsObP5cUWJzzMaimVRsST21W33JDg6/view

iv) NEW Data_CSV_Vehicle_Count: https://drive.google.com/file/d/1CG5WK-f8EYY3g6lrbF31cSjpcMJrF_fw/view

v) COMBINED_Data_CSV_Vehicle_Count:

https://drive.google.com/drive/u/0/folders/1_GmS-Ki602h1yeITp1hbE4XD7Y_Hk1iL

vi) frequency:

https://drive.google.com/file/d/1guf6-7V_1WwIngumUS7pfMXlI2KkCT86/view

vii) velocity:

https://drive.google.com/file/d/1AGGC3BwVOvdLwx4KxMjfwl1mIQFEOAxW/view

We calculated frequencies at node A and Node C on April 19 and 21, respectively. But as our school starts at 7:15 am and we were a bit late, actually, we recollected the frequency on June 6.
Overall, for Node A and C, the speed calculation was done on 19 and 21 April, while the frequency was considered from June 6.

The Reviewer’s Critique: "Traffic patterns, weather conditions (e.g., June monsoons in Dhaka), and school attendance dynamics change between April and June. How can you combine them?"

Your Academic Defense: I will add this in the "Limitations of the Study" section. While absolute volumes might vary across months, the speed-to-volume ratio per vehicle class remains structurally constant within the narrow 7:15 AM school-drop-off window. The weighted velocity mechanism acts as a mathematical stabilizer against these baseline shifts

1)

Consider the frequency file as NEW Data_CSV_Vehicle_Count and velocity.

# we have two files. At first, we will cross-check the vehicle names and time slot names

# We also need to check the PCU
Code: https://github.com/sakhorimtiaz/Modelling-Morning-Traffic-Around-School/blob/Think-Python-by-Allen-B.-Downey/step1.py

Output:
C:\Users\THINKPAD\PycharmProjects\python_for_data_analysis\pythonProject7\Scripts\python.exe C:\Users\THINKPAD\PycharmProjects\pythonProject7\traffic13.py

Frequency Vehicles: {'Truck', 'Leguna', 'PrivateCar', 'Bicycle', 'MotorBike', 'Van', 'AutoRickshaw', 'ManualRickshaw', 'CNG'}

Velocity Vehicles: {'Scooter', 'Leguna', 'PrivateCar', 'Bicycle', 'MotorBike', 'Rickshaw', 'AutoRickshaw', 'ManualRickshaw', 'Pickup'}

Process finished with exit code 0

2)

# CNG and AutoRickshaw have almost the same PCUs

# Although we don't have the measurement of Pickup, let’s consider 'Pickup' = 'Leguna' and 'CNG': 'AutoRickshaw

# We have also renamed some column and row names,,,

# Then we calculated row-wise total PCU by multiplying frequency with PCU

Code: https://github.com/sakhorimtiaz/Modelling-Morning-Traffic-Around-School/blob/Think-Python-by-Allen-B.-Downey/step2.py

Output:

frequency_cleaned:

https://drive.google.com/file/d/1QZeUK1qvYEFdU4IciQoCQHyZSPZZpoKL/view

velocity_cleaned:

https://drive.google.com/file/d/1PbbWbygr2dsExiywyZKUOPWwwUACMnb0/view

3)

Lets calculate PCU per hour

Code:

https://github.com/sakhorimtiaz/Modelling-Morning-Traffic-Around-School/blob/Think-Python-by-Allen-B.-Downey/step3.py

Output:

flow_rate_summary:
https://drive.google.com/file/d/1v9TN_awMJ6FpxdEW7t9wE0KvLZ5GT5HH/view

4)

Now let’s organize the frequency_cleaned file according to this

Node > TimeSlot > VehicleType > Vehicle > Total_PCU

Code:

https://github.com/sakhorimtiaz/Modelling-Morning-Traffic-Around-School/blob/Think-Python-by-Allen-B.-Downey/step4.py

Output:

frequency_cleaned_organized:
https://drive.google.com/file/d/1XH1tBmz-j8ZCJVgKVZP_-10I2cBtFt3M/view

5)

After changing the names from CNG to AutoRickshaw we have two rows having same vehicle names but different values. We need to combine them.

Code:

https://github.com/sakhorimtiaz/Modelling-Morning-Traffic-Around-School/blob/Think-Python-by-Allen-B.-Downey/step5.py

Output:
frequency_pcu_combined

https://drive.google.com/file/d/1ivzDJJ1_zjGTOfyHau4Z5JmFeddqLP1B/view

6)

In the frequency_pcu_combined file we have two or more values of same vehicles.

We will calculate the average.

Code:

https://github.com/sakhorimtiaz/Modelling-Morning-Traffic-Around-School/blob/Think-Python-by-Allen-B.-Downey/step6.py

Output:
speed_averaged_organized

https://drive.google.com/file/d/1I4QVR6GV63DHOwDonuvSV_qFrLHT0Gd8/view

7)

We will calculate the weighted velocity.

We will select a Node > TimeSlot > Vehicle > Speed from the speed_averaged_organized file.

Then multiply the speed by the Total_PCU from the frequency_pcu_combined file.

Repeat the process for other vehicles for the speed_averaged_organized file.

Loop until all the vehicles for a particular Node and TimeSlot are done.
Then sum the values and, at last, divide by the total PCUs in a particular node and time.

[Note that, for Node A in the Morning, in the speed_averaged_organized file we have measured velocity of a Bicycle is 25.09, while frequency_pcu_combined shows no Bicycle.
It is because we calculated the speed and frequency of Node A and Node C for the morning slot on two different days. 
However, this does not affect our result, as we are using weighted velocity.

Because the weight (PCU) is zero, it completely zeros out in the macro-velocity equation, leaving your final metrics mathematically clean.

Code:
https://github.com/sakhorimtiaz/Modelling-Morning-Traffic-Around-School/blob/Think-Python-by-Allen-B.-Downey/step7.py

Output:
macroscopic_velocity

https://drive.google.com/file/d/1UvW_w-3w0WYTf076daNfuhJQhNx8cHpG/view

8)

# now we will calculate the traffic flow density (k)

# by dividing PCU/hr by Average_Velocity.

Code:
https://github.com/sakhorimtiaz/Modelling-Morning-Traffic-Around-School/blob/Think-Python-by-Allen-B.-Downey/step8.py

Output:
traffic_flow_density

https://drive.google.com/file/d/1PuBaua-grlwwiAmoTpMk36SrQv1_3PKw/view

9)

Let’s check the bar diagram

Code:
https://github.com/sakhorimtiaz/Modelling-Morning-Traffic-Around-School/blob/Think-Python-by-Allen-B.-Downey/step9.py

Output:

We have one data point missing

10)

# As we can see that the Average velocity of Node B Morning is missing in the macroscopic_velocity file, the density (k) in the output for Node B Morning density (k) is also missing.

#We are considering the average velocity of evening in Node B as Free flow

# Let's put the value and create a new file

[Node B Evening operated at an incredibly low flow (q = 275.36 PCU/hr) and low density (k = 26.93 PCU/km), proving it represents a true "Uncongested Base State." Substituting this value provides a conservative baseline for the morning model, preventing overestimation of congestion.]

Code:
https://github.com/sakhorimtiaz/Modelling-Morning-Traffic-Around-School/blob/Think-Python-by-Allen-B.-Downey/step10.py

Output:
macroscopic_velocity_updated

https://drive.google.com/file/d/1pezF92htGryd09t0InVad31QNgB818ZJ/view

11)

Now calculate the traffic density again

Code:

https://github.com/sakhorimtiaz/Modelling-Morning-Traffic-Around-School/blob/Think-Python-by-Allen-B.-Downey/step11.py

Output:
https://drive.google.com/file/d/14IeMfkMAthVHZXGf2Paa6IIS5BcFBf9S/view

Output bar chart:
https://docs.google.com/spreadsheets/d/1cr6Y7wnTdLvHQzgn_fJnhr-85gyiUIw7MiVFPmK7LFo/edit?gid=0#gid=0

12)

Now we will draw the scatter chart:
Code:
https://github.com/sakhorimtiaz/Modelling-Morning-Traffic-Around-School/blob/Think-Python-by-Allen-B.-Downey/step12.py

Scatter chart in Google Sheets

https://docs.google.com/spreadsheets/d/1YsxCwK0eRatjjFcVBJuMLIp2l4Nu7LM6evMrVzlCarM/edit?gid=452756176#gid=452756176

How to draw a scatter chart and what does a scatter chart represent?

https://docs.google.com/document/d/1DXmm2BNhQ79dyx-r_o5uKWO632n2OCaewYivxkjkn70/edit?tab=t.0

13)

# now we will solve the equation found in the scatter chart.

# then create a new file

Code:

https://github.com/sakhorimtiaz/Modelling-Morning-Traffic-Around-School/blob/Think-Python-by-Allen-B.-Downey/step13.py

Output:

traffic_flow_analysis_results

https://drive.google.com/file/d/1K6WoBYyGXwpesupOqbGs_H9NE_eJzDju/view

<<<<<<>>>>>>>>

14)

LOS

LOS_ground_truth:

https://docs.google.com/document/d/1KylFipzWFkRGrCs8kBzsTOmUwScynTsajEb-LJrDFn4/edit?tab=t.0

Output:
level_of_service_LOS

https://docs.google.com/spreadsheets/d/1FtYLK5TmLO9OHhjcIkSKI7_BfEDabe2La0MU1jzUt14/edit?gid=953751429#gid=953751429

15)

Percentage to road occupied:
Output:
percent_of_road_occupied

https://docs.google.com/spreadsheets/d/1pLemIE38RaPIpmWGBV5K-4WNcfkCGMnPUo2wERpCsdA/edit?gid=289363197#gid=289363197

16)

Queue length

[unfinished]

17)

Effective weight.

[Unfinished]