Look at the table :  level_of_service_LOS.csv

Here are the two reference charts, cleanly separated so a reader can instantly see the difference between the textbook formula logic and your finalized scientific tier system.

### Reference Chart I: Python Code Grades (Density Only)

This chart shows the exact mathematical limits hard-coded into the Python script. It grades the road based strictly on physical space (Density), completely ignoring how fast the traffic is actually moving or how many total vehicles are passing through.

| Code Grade (LOS) | Density Range (k) | Short Explanation (The Code's Logic) |
| --- | --- | --- |
| A (Free Flow) | 0 to 15 PCU/km | The road is virtually empty. Vehicles have complete freedom to maneuver. |
| B (Reasonably Free) | 15.1 to 25 PCU/km | Traffic is light. The code assumes vehicles are still moving at high speeds without interacting much. |
| C (Stable Flow) | 25.1 to 35 PCU/km | Moderate traffic. The code assumes lanes are filling up, but the flow remains perfectly stable. |
| D (Approaching Unstable) | 35.1 to 45 PCU/km | Heavy traffic. The code assumes space is running out and any sudden braking could cause a minor jam. |
| E (The Breakpoint) | 45.1 to 65 PCU/km | Maximum textbook capacity. The code assumes the road cannot hold any more vehicles before completely collapsing. |
| F (System Failure) | Greater than 65 PCU/km | Textbook gridlock. The code assumes density has crossed the critical breakpoint, resulting in zero movement and total system failure. |

### Reference Chart II: Multi-Dimensional Scientific Tiers (Holistic Reality)

This is your final scientific framework. Unlike the code above, these tiers analyze the complete urban reality by combining Flow, Density, and Velocity, proving that mixed traffic behaves differently than textbook highway models.

Tier 1: Uncongested Regime (Corresponds to standard LOS A – C)

- Definition: A free-flowing state where vehicle density is low and operational speeds are high. Because total traffic volume is so light, it falls below the mathematical "floor" of our aggregate model (283 PCU/hr), meaning a master equation cannot accurately map this specific state.

- Data Ranges:

  - Flow ($q$): Less than 300 PCU/hr

  - Density ($k$): Less than 30 PCU/km

  - Speed ($V$): 10 to 19 km/h

Tier 2: Transition Peak Regime (Corresponds to standard LOS D – E)

- Definition: A highly efficient, stable state of high-volume capacity. Vehicles fill the lanes, but because of mixed-mode spatial efficiency (e.g., motorcycles and vans sharing lanes), the traffic stream maintains moderate speeds and excellent throughput without collapsing.

- Data Ranges:

  - Flow ($q$): 400 to 600 PCU/hr

  - Density ($k$): 30 to 60 PCU/km

  - Speed ($V$): 10 to 16 km/h

Tier 3: Urban Forced Creep Regime (Corresponds to standard LOS F)

- Definition: A state of extreme urban congestion where physical space is completely filled. While textbook theory dictates this should cause a total system failure (zero flow), mixed urban traffic adapts. The result is a severe speed bottleneck (walking pace), but the system refuses to lock up, allowing massive volumes of traffic to continuously squeeze forward.

- Data Ranges:

  - Flow ($q$): Greater than 550 PCU/hr

  - Density ($k$): Greater than 90 PCU/km

  - Speed ($V$): Less than 6 km/h